ODU 3rd Raspberry Pi Contest – Fall 2022

Emulation Station + Custom Arcade Stick

Project Report

By: Sean Allgaier

[Note: The references notes are for specific images in the provided PowerPoint]

Emulation Station:

- Hardware:

  - Raspberry Pi 4b 8gb ram

  - Eviciv 7” touchscreen display case for the Pi

  - SanDisk 256gb microSD

  - Aluminum heatsink w/ fans to add extra cooling in the display case

  - Xiwai CYFPV 90-degree micro-HDMI male to HDMI male ribbon cable

  - Other hardware used will be described within presentation and demo videos

- Software:

  - Recalbox:

    - Raspberry Pi OS designed to run EmulationStation, which is an emulator front-end that contains different emulators for retro video game consoles and removes the Pi’s requirement for a keyboard.

    - Designed to be an “All-in-One” retro gaming console, according to their website: https://www.recalbox.com/

- Hardware Difficulties/Challenges:

  - The Raspberry Pi 4b has 2 female micro-HDMI ports which can be used to send the Pi’s video/audio feed to two different screens simultaneously, so there was an attempt to use one of the ports for the adapter that came with the display case and the other for the HDMI ribbon cable so the Pi could output video to the touch screen and another external screen at the same time. However, the micro-HDMI adapter used by the case was slightly too wide to accommodate both it and the ribbon cable. [Reference ESHC1.0]

  - The display case has a feature that allows it to be used as a monitor for other HDMI devices via an external HDMI port. However, the external HDMI port only allowed for an input HDMI feed, not output, so if the internal micro-HDMI adapter was used instead of the ribbon cable, the HDMI feed of the Raspberry Pi wouldn’t be accessible to any other external screens.

  - Originally the decision was made to use the touch screen’s internal micro-HDMI adapter because it had a cleaner look, but later on it was changed to use the ribbon cable instead because it allowed for external monitors to be used, which made it easier for demonstration purposes due to the screen’s small size making it harder to see. [Reference ESHC1.1]

  - The ribbon cable was fed through one of openings on the side of the display, one of which was designed for the USB plug required for the screen’s touch feature. Thankfully the other opening was large enough to accommodate the ribbon cable. [Reference ESHC1.2]

- Software Difficulties/Challenges:

  - Originally wanted to use the highly customizable OS called Retropie, which also runs EmulationStation, similarly to Recalbox. However, every time there was an attempt to install and update all of the drivers, the Pi would crash, and OS would become unusable. The fix for which required a fresh install of the OS, which meant the entire process of installing and updating drivers would also have to be restarted. Eventually, the decision was made to use Recal Box instead because it was more stable.

Custom Arcade Stick:

- Hardware:

  - Sanwa JLF-TP Arcade Joystick

  - EG STARTS LED Arcade Buttons (x10)

  - Paradise Arcade LED Arcade Buttons (x7)

  - 6” Micro-USB cable

  - Raspberry Pi Pico

  - Treedix Breakout Board w/ Pin Headers for Pi Pico

  - USB A female to USB B female adapter

  - USB B male to USB A male cable

  - Arcity 6.3mm terminal wires for daisy chaining together button LEDs

  - Easyget Generic Zero Delay Game Controller w/ 4.88mm terminal wires

  - Other hardware used will be described within presentation and demo videos

- Software:

  - GP2040-CE

    - Gamepad firmware for the Raspberry Pi Pico and other boards based on the RP2040 microcontroller. Allows for custom button schemes across multiple platforms as well as some other features.

    - Website: https://gp2040.info/#/

- Hardware Difficulties/Challenges:

  - Before starting the project, I had a general idea of a plan. I originally wanted to make this project back in Spring of 2019 but didn’t have the funds to do more than 3D-print a custom case for my Raspberry Pi 2b. Then in April of 2021 a friend came across a tutorial video to make a controller for a relatively cheap price, and we decided to team up to make controllers. The plan ended up falling out due to an inability to commit the time or the funds. When the opportunity for this competition came up, I used the video my friend found as inspiration to make my controller. The video uses an Arduino to control everything and keyboard keys as the buttons. https://www.youtube.com/watch?v=trGa98tEy_0&ab_channel=Crane%27sLab

  - I had an Arduino nano laying around somewhere from a previous project of mine, however, I was unable to find it, so I went ahead and purchased a Raspberry Pi Pico instead of a new Arduino because it was cheaper.

  - I wanted a more traditional arcade feel for my controller, so I went with standard arcade buttons with LEDs, and I chose a Sanwa joystick because they are traditionally used in arcade cabinets.

  - I don’t have much experience with making plans, so I had a decent amount of trouble with drawing decent schematics that could be translated into creating a box out of wood to contain all of the components, while still retaining the functionality of being solid and can be opened in the need of any form of maintenance.

  - I also don’t have much experience with wood projects, however, my dad has a plethora of experience and tools for working on wood projects, so I asked him to help me with building the box.

- Software Difficulties/Challenges:

  - The biggest challenge for the software was the time I had remaining. By the time the box was made finished, I had only installed the gamepad software and customized which pin on the Pi Pico represented which button. Originally, I wanted to use this software simply to test the buttons then make my own program in C++ to read the button inputs, however, I ran out of time before the final presentation date to make my own controller software.

All other hardware/software information is covered in the demo and explanation videos.